OBD2 PERFORMANCE LOGGER M5Stack Tab5 · ESP32-P4 · CAN Bus · GPS 10 Hz · Arduino · M5GFX Sprites · 3-Bar MAP Professional drag timing, dual gauge cluster with peak tracking, ISO 15765-2 DTC scanner — built for the track.

| LIGHT TREE |  | BOOST + RPM |  | PEAK TRACKERS |  | 3-BAR MAP |  | GPS 10 Hz |  | DTC SCANNER |  | SPRITE UI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Launch System & Light Tree

| Professional Drag Light Tree. ▸ Full NHRA-style 3-amber → green countdown, 500 ms per stage. ▸ Auto-stages when GPS fix acquired, 4+ satellites locked, speed < 3 mph. ▸ Overlaid directly on gauge centre — no separate screen needed. ▸ Tap screen to start sequence once staged; green shows GO! / TAP. ▸ Un-stages automatically if GPS fix is lost or car moves before sequence starts. ▸ OBD2 polling suspended during countdown (AMBER→GREEN) to prevent CAN timeouts disrupting timing. |  | Reaction Time & Launch Detection. ▸ Reaction time measured from green light to IMU threshold crossing. ▸ Tap at green also triggers launch and records reaction time. ▸ IMU axis selectable: X / Y / Z to match physical mounting orientation. ▸ Launch threshold: 0.05–0.35 g in 0.05 g steps (default 0.10 g). ▸ Live IMU readout in Settings shows all 3 axes updating in real time. ▸ Bar graph with threshold tick mark — move device forward to identify correct axis instantly. |
| --- | --- | --- |

Dual Gauge Cluster

Boost Gauge — Centre Arc

▸ Range: −15 to +30 PSI — extended for 3-bar MAP sensor headroom (was −15 to +20 PSI)

▸ Overboost zone starts at 22 PSI — needle and value turn red above this threshold

▸ All range values defined as named constants: BOOST_PSI_MIN, BOOST_PSI_MAX, BOOST_PSI_RANGE, BOOST_OVERBOOST

▸ Coloured arc background: blue-grey (vacuum) / amber (boost) / red (overboost) — smooth anti-aliased bands via M5GFX drawArc()

▸ Live arc sweeps from zero to current PSI in real time — like a real instrument cluster

▸ Large centre numeric readout: green (vacuum) / amber (boost) / red (overboost)

▸ Peak boost label PK xx.x shown above centre value during and after run

▸ Rendered into 600×600 M5GFX sprite, pushed atomically — zero flicker, no needle ghost artifacts

RPM Gauge — Outer Concentric Arc

▸ Same 260° sweep, range 0–8000 RPM — both arcs share the same centre and geometry

▸ Colour zones: grey (normal) → amber (1000 RPM before redline) → red (at/above redline)

▸ Cyan live needle and fill arc sweep from 0 to current RPM, turn red above redline

▸ RPM redline configurable in settings: 4000–9000 RPM in 500 RPM steps

▸ Peak RPM marker — bright cyan tick latches at highest RPM seen since last reset

▸ Peak RPM value printed as small number at the arc outside the marker position

▸ Both peak markers (boost + RPM) reset on tap-to-reset; persist across finish banner

Performance Metrics

| Quarter-Mile Timing. ▸ GPS path distance accumulation — actual driven distance, not straight-line. ▸ 10 Hz GPS: ±4.5 m position error at finish (~0.1 s at 100 mph). ▸ 15 m glitch filter eliminates GPS multipath jumps at speed. ▸ Finish time and trap speed frozen for post-run review. |  | 0–60 & Elapsed Timer. ▸ 0–60 captured automatically from GPS speed crossing 60 mph. ▸ Elapsed timer starts at IMU launch detect, not tree green. ▸ Both captured mid-run — independent of each other. ▸ Elapsed freezes at quarter-mile; 0–60 persists until reset. |
| --- | --- | --- |
| Peak Boost Tracker. ▸ Running maximum PSI tracked from launch to finish. ▸ Shown as PK xx.x above live boost value during and after run. ▸ Logged in CSV as a dedicated column alongside live boost. ▸ Resets on each new launch — clean slate for every pass. |  | Peak RPM Tracker. ▸ Maximum RPM since last reset tracked continuously. ▸ Cyan tick mark on outer RPM arc shows peak position visually. ▸ Numeric value printed at arc outside the tick. ▸ Persists after run ends — visible on gauge until tap-to-reset. |

OBD2 Telemetry & CAN Bus

| Live OBD2 Data Channels. ▸ MAP — Manifold Absolute Pressure → Boost PSI (kPa − baro × 0.145038) — tuned for 3-bar sensor. ▸ RPM — Engine speed, two-byte PID, full resolution. ▸ IAT — Intake Air Temp, raw OBD2 → °F with warn/crit colour thresholds. ▸ BARO — Barometric pressure at boot, 70–110 kPa validation, used as boost zero reference. ▸ OBD2 skipped during tree countdown to prevent CAN timeouts stalling the sequencer. |  | CAN Bus Reliability. ▸ Non-blocking transmit (0-tick timeout) — full TX queue never freezes the loop. ▸ 200 ms per-PID receive timeout, 50 ms poll steps. ▸ Automatic TWAI driver recovery: up to 5 stop/uninstall/reinstall cycles. ▸ 5-second health check; triggers if no frames received in 10 seconds. ▸ CAN recovery display guarded — does not corrupt settings or DTC screens. |
| --- | --- | --- |

Diagnostic Code Scanner

Full OBD2 Mode 03 / Mode 04 Implementation

▸ Access via long-press → Settings → SCAN / CLEAR CODES

▸ Mode 03 with full ISO 15765-2 multi-frame: Single Frame, First Frame, Flow Control, Consecutive Frames — handles any number of stored codes

▸ Mode 04 clear with two-tap confirmation — first tap arms red confirm state, second tap sends clear, auto-rescans

▸ 584-entry PROGMEM binary-search lookup table: P0xxx–P3xxx, B0xxx–B1xxx, C0xxx, U0xxx

▸ Full descriptions: all misfires, O2 sensors (Banks 1–4), MAF/MAP, injector circuits, knock, cam/crank sensors, VVT/VANOS, turbo/wastegate, EVAP, catalyst, EGR, idle, VSS, transmission solenoids A–E, ABS wheel speed, airbag squib circuits, network loss codes

▸ Colour-coded cards: Amber = Powertrain, Cyan = Body, Green = Chassis, Yellow = Network

▸ Scrollable list — tap upper/lower half to scroll; arrows shown when overflow exists

▸ CLEAR CODES greyed and inactive when no codes present

Settings Screen

In-App Settings — Long Press 1.5 s to Open

▸ LAUNCH AXIS (IMU) — cycle X / Y / Z to match physical mounting

▸ LAUNCH THRESHOLD — 0.05 / 0.10 / 0.15 / 0.20 / 0.25 / 0.30 / 0.35 g (default 0.10 g)

▸ RPM REDLINE — 4000–9000 RPM in 500 RPM steps (default 6000)

▸ IAT WARN TEMP — 100–160 °F in 10 °F steps (default 120 °F)

▸ IAT CRIT TEMP — warn+10 to 200 °F in 10 °F steps (default 150 °F)

▸ SCAN / CLEAR CODES — navigates directly to DTC scanner screen

▸ Live IMU readout: three columns (X / Y / Z) updating at loop rate with bar graphs and threshold tick mark

▸ Selected axis highlighted with amber border — move device forward to confirm correct axis

▸ All settings take effect immediately on BACK — no save step needed

Display, UI & Sprites

| M5GFX Sprite Architecture. ▸ Three M5Canvas sprites: gaugeSprite (600×600), leftSprite (200×panel), rightSprite (200×panel). ▸ All sprites allocated in PSRAM — no DRAM impact on main stack. ▸ Gauge pushed atomically each frame via DMA — zero screen tearing. ▸ Left and right panels rebuilt and pushed each loop — no partial-update glitches. ▸ Anti-aliased arc bands via drawArc() — smooth edges vs old pixel-staircase tick lines. |  | Layout — 1280 × 720. ▸ Status bar (32 px top): GPS sats · OBD2 LIVE/OFF · SD + battery voltage. ▸ Left panel (200 px): Elapsed / reaction time / 0–60 / 1/4 mile / trap speed. ▸ Centre: dual concentric gauge arcs with live + peak needles and light tree overlay. ▸ Right panel (200 px): RPM / GPS speed / IAT °F in three sections. ▸ Bottom banner (40 px): contextual run state, colour-coded per state. |
| --- | --- | --- |
| Boot Splash Screen. ▸ Dark charcoal background with amber accent top bar. ▸ Live 5-row checklist: IMU · SD · CAN · OBD2 · Baro — dot + OK/FAIL per step. ▸ SD shows card size (MB) on success; Baro shows reading (kPa) on success. ▸ Remains visible 1.2 s after final step. |  | Screen Dimming & Touch. ▸ Dims to brightness 60/255 after 5 minutes idle. ▸ Any touch restores full brightness — wake touch consumed, no accidental trigger. ▸ Short tap: start tree (staged), launch (green), or reset (any other state). ▸ Long press 1.5 s: open Settings from main screen. ▸ 500 ms debounce between reset events prevents accidental double-reset. |

SD Card Data Logging

CSV Run Logging

▸ SDIO 4-bit interface — Tab5 native pins (GPIO 39–44)

▸ Auto-numbered run files: run_1.csv, run_2.csv … run_999.csv

▸ CSV columns: Time_s, GPS_Speed_mph, RPM, Boost_psi, Peak_Boost_psi, IAT_F, Distance_ft, Lat, Lng

▸ File opened on launch detect, flushed and closed at quarter-mile finish or manual reset

▸ 10-record RAM buffer, flush to SD every ~1 second — reduces seek overhead

▸ SD write failure handled cleanly — logging latched off without corrupting file or crashing

▸ Run number shown in launch banner so you always know which file is being written

Hardware & Technical Specs

| Platform | M5Stack Tab5 · ESP32-P4 |
| --- | --- |
| Display | 1280 × 720 · Landscape |
| MAP Sensor | 3-bar (tuned) · PID 0x0B · Boost range −15 to +30 PSI |
| Overboost Threshold | 22 PSI (BOOST_OVERBOOST constant) |
| CAN Interface | TWAI (ESP32 native) · 500 kbps · ISO 15765-2 |
| CAN Pins | TX: GPIO 49 · RX: GPIO 50 (Expansion Header) |
| GPS Interface | HardwareSerial(1) · HY2.0-4P Grove Port |
| GPS Pins | RX: GPIO 53 (Yellow) · TX: GPIO 54 (White) |
| GPS Rate | 10 Hz (MTK / CASIC negotiated at boot → 115200 baud) |
| IMU | BMI270 via M5Unified · Axis: X/Y/Z selectable |
| SD Interface | SDIO 4-bit · SD_MMC · GPIO 39–44 |
| Battery | NP-F550 7.4V · Warn <7.0V · Crit <6.5V |
| Sprite Memory | gaugeSprite 600×600 + 2× panel sprites · ~800 KB PSRAM |
| Main Loop Rate | ~100 Hz (10 ms delay) · OBD2 limits effective rate |
| SD Log Rate | 1 row/second · 10-record buffer |
| GPS Glitch Filter | 15 m max segment at 10 Hz |
| CAN Timeout | 200 ms per request · 0-tick non-blocking transmit |
| CAN Recovery | Up to 5 automatic TWAI driver restarts |
| Screen Dim | Brightness 60 after 5 min idle · Full on touch |
| Long Press | 1500 ms to open Settings |
| Tree Stage Time | 500 ms per amber stage |
| DTC Table | 584 entries · PROGMEM · Binary search |
| Build | Arduino · M5Unified · M5GFX · TinyGPS++ · TWAI |

OBD2 Performance Logger  ·  M5Stack Tab5  ·  Arduino  ·  3-Bar MAP Edition